Attachment C

SAFETY PLAN REQUIREMENT CHECKOFF LIST FOR GENERAL CONTRACTORS

The Contractor’s Safety Program MUST incorporate the following HISD safety requirements with all applicable, current OSHA Standards.  The Proposer shall execute this form to certify that the Contractor’s Safety Plan meets and/or exceeds HISD’s safety requirements.

SAFETY PLAN REQUIREMENT CHECK LIST FOR GENERAL CONTRACTORS

# OSHA REQUIREMENTS

Current Safety Program Documentation must be and has been provided for this RFQ.

Conform to OSHA Requirements of 29CFR1926 & applicable section of 29CFR1910

Conform to all State & Local Regulations & Requirements that are applicable

# ABOVE-OSHA REQUIREMENTS

Accident Reporting - Must be timely & properly documented within 24 hours of incident

“Baker” Scaffolding - Fall protection required at 4’ above working surface

Clothing - Shirts with sleeves, long pants, substantial work shoes or boots

Confined Spaces - Procedures, testing, & training in accordance with OSHA 1910.146

Controlled Substances (including alcohol) - Prohibited from the job site

Cranes - Independent 3rd party inspection by agency accredited by USDOL OSHA under the regulations of Title 29, Chapter XVIII, Part 1919

Discipline Program - Verbal Warning, Written Warning, Dismissal

Electrical Wiring - Ground Fault Circuit Interrupters on all temporary power wiring

Electrical Cords & Tools - Assured Equipment Grounding Program required

Emergency Procedures - Compliance mandatory

Fall Protection - Required starting at Six Feet above working surface

Fall Protection (Steel Erection) - 100% Tie off shall be primary means of protection

Fall Protection (Precast Concrete) - 100% Tie off shall be primary means of protection

Attachment C

SAFETY PLAN REQUIREMENT CHECKOFF LIST FOR GENERAL CONTRACTORS

(Continued)

Fall Protection (Written Fall Protection Programs) - Will be required; No Exceptions

Fighting - Prohibited on the job site; offenders shall be dismissed immediately

Hazard Communication - HazCom Programs & Material Safety Data Sheets required

Hot Work - Permits and dedicated Fire Watch required for all operations.

Housekeeping - Combustible scrap materials & debris must be removed daily.

Immediate Danger to Safety or Health - Work shall be stopped immediately

Job Hazard Analysis - Must be created prior to & strictly followed for all work activities

Orientation Session - All employees must attend before starting work at Project Site

Personal Protective Equipment - Hard Hats & Safety Glasses required at all times

Principal’s Meeting - Must be performed within 72 hours after a Lost Time Accident

Return to Work - Restricted Duty to be made available to extent allowed by Doctor

Safety Representative - Must meet requirements of the Competent Person Standard and have completed OSHA 10 hour construction outreach training.

Scaffolding - Inspection & Documentation required daily by Competent Person

Training - Weekly “Tool-Box” Safety Meeting performed with attendance roster

Weapons - Possession of weapons on the project site is prohibited

Weekly Safety inspections conducted by Safety Representative

Contractor has included one (1) copy of their current Safety Manual with this proposal

I certify that our company’s Safety Plan meets and/or exceeds the HISD Safety Requirements on this checklist.

Corporate Officer:

Company:

Date: